## 1. Creating an Extended ACL to Block Botnet Command-and-Control (C2) Traffic

The following extended Access Control List (ACL) is configured to deny Telnet (TCP port 23) traffic originating from known botnet command-and-control (C2) hosts while allowing all other IP traffic.

ip access-list extended BLOCK_BOTNET_C2
 deny tcp host 219.243.27.65 any eq 23
 deny tcp host 71.181.6.40 any eq 23
 permit ip any any

## 2. Applying the ACL to the Gateway Interface

The ACL is applied inbound on the gateway interface to inspect and filter incoming traffic before it enters the network.

interface GigabitEthernet0/0
 ip access-group BLOCK_BOTNET_C2 in

## 3. Verifying ACL Operation and Match Statistics

The ACL hit counters confirm that traffic from the identified malicious hosts has been successfully blocked. The match counts indicate the number of packets denied or permitted by each ACL entry.

Router# show access-lists BLOCK_BOTNET_C2

Extended IP access list BLOCK_BOTNET_C2
    10 deny tcp host 219.243.27.65 any eq 23 (1452 matches)
    20 deny tcp host 71.181.6.40 any eq 23 (894 matches)
    30 permit ip any any (52341 matches)

Result: The ACL effectively prevents Telnet-based communication from the specified botnet C2 addresses while permitting legitimate network traffic to continue uninterrupted.