### Network Address Translation (NAT) Gateway Configuration

The following NAT configuration enables devices on the internal network to access external networks through a single public IP address. Port Address Translation (PAT), also known as NAT overload, is used to allow multiple internal hosts to share the WAN interface address.

interface GigabitEthernet0/0
 description Internet-Facing WAN Interface
 ip address dynamic
 ip nat outside
!
interface GigabitEthernet0/1
 description Local Home Network Gateway
 ip address 192.168.100.1 255.255.255.0
 ip nat inside
!
ip nat inside source list 1 interface GigabitEthernet0/0 overload
access-list 1 permit 192.168.100.0 0.0.0.255

### Configuration Overview

- GigabitEthernet0/0 is designated as the external (outside) interface connected to the Internet and obtains its IP address dynamically from the service provider.

- GigabitEthernet0/1 serves as the internal (inside) gateway for devices on the 192.168.100.0/24 network.

- NAT overload translates multiple private IP addresses to the public IP address assigned to the WAN interface, allowing concurrent Internet access.

- Access Control List (ACL) 1 identifies the internal subnet whose traffic is eligible for address translation.

### Operational Purpose

This configuration preserves secure connectivity between the local network and the Internet while conserving public IP address usage. By translating private addresses at the network boundary, the gateway enables outbound communication for all authorized hosts on the internal subnet while maintaining a simplified and scalable network design.