### Layer 2 Switch Port Security Configuration

The following switch port security configuration restricts access to a single authorized device by binding the port to a specific MAC address. Any attempt by an unauthorized device to connect will trigger a security violation and automatically disable the port.

interface FastEthernet0/10
 description Secure Link to Raspberry Pi
 switchport mode access
 switchport access vlan 20
 switchport port-security
 switchport port-security maximum 1
 switchport port-security mac-address b827.ebc7.e9c9
 switchport port-security violation shutdown

### Port Security Verification

The verification output confirms that port security is active on FastEthernet0/10 and that the port is operating in a secure state. Only the designated MAC address is permitted to use the interface.

Switch# show port-security interface fa0/10

Port Security              : Enabled
Port Status                : Secure-up
Violation Mode             : Shutdown
Maximum MAC addresses      : 1
Total MAC addresses        : 1
Configured MAC addresses   : 1 (b827.ebc7.e9c9)

### Security Outcome

The configuration successfully enforces Layer 2 access control by allowing connectivity only for the registered Raspberry Pi device. If a different device attempts to connect through FastEthernet0/10, the switch will place the port into a shutdown state, preventing unauthorized network access and mitigating risks such as rogue device connections and MAC spoofing attempts.